Chemistry Investigatory Project – Fuels in F1

Basics:

What are fuels?

Ans: Fuels are materials that store energy and can release it in a usable form, typically as heat energy, when they react with other substances (most commonly through combustion or burning). This released energy can then be used for various purposes like heating, cooking, transportation, or generating electricity.

In a chemical fuel, the process of combustion involves the atoms (primarily carbon and hydrogen) in the fuel combining with oxygen in the air, resulting in a chemical reaction that releases energy in the form of heat and light.

The Core Chemical Reaction: Combustion

The majority of fuels, particularly fossil fuels and biofuels, are organic substances composed mainly of hydrocarbons (molecules containing only hydrogen and carbon atoms).

The General Equation

In a complete combustion reaction, a hydrocarbon fuel reacts with oxygen to form carbon dioxide releasing a large amount of energy (heat and light)

Combustion plays a vital role in many everyday applications, from household uses to transportation. One of the most advanced and high-performance examples of combustion can be seen in motorsport — particularly in Formula 1. In these cars, combustion isn’t just about producing energy; it’s a finely tuned chemical process that determines the car’s speed, efficiency, and overall performance.

The chemistry of fuels used in Formula 1 (F1) cars is a highly specialized and tightly regulated subset of gasoline chemistry, focused on maximizing energy release, thermal efficiency, and, increasingly, sustainability.

⛽ Current F1 Fuel Chemistry (E10)

Since the 2022 season, F1 cars run on E10 fuel, a mandated blend that brings the chemistry closer to commercial pump fuels while demanding high performance.

- E10 Blend: The fuel consists of approximately 90% unleaded gasoline and 10% renewable ethanol.

- Gasoline Base: The gasoline is a complex mixture of various hydrocarbons (alkanes, cycloalkanes, and aromatics), similar to premium road-car petrol. However, the specific ratio and structure of these hydrocarbons are intensely optimized.

- Renewable Ethanol C2H5OH : This is a simple alcohol that acts as an oxygenate, meaning it contains oxygen. The O atom in ethanol helps ensure more complete combustion, increasing the thermal efficiency of the engine. The ethanol used in F1 must be a second-generation biofuel, produced sustainably from non-food biomass (like waste or straw) to meet environmental goals.

🔥 Performance Optimization and Additives

While F1 fuel must resemble commercial gasoline (it cannot contain compounds not generally found in commercial fuel), its chemistry is fine-tuned for the extreme conditions of a modern F1 engine:

- Octane Rating: F1 fuels have a very high Research Octane Number (RON), often around 98 to 102, compared to standard pump fuel. A high octane rating is crucial because it indicates the fuel's resistance to knock (premature, uncontrolled ignition caused by high compression). The high-compression, turbocharged F1 engines demand this knock resistance to operate at peak efficiency and power without damage.

- Custom Blends: Each team works closely with its fuel partner (e.g., Shell, Petronas) to create a bespoke formulation. They adjust the precise mix of hydrocarbon isomers and specialized additives to perfectly match the combustion characteristics and thermal dynamics of their unique engine.

- Vaporization and Volatility: The fuel is engineered for optimal volatility—how easily it turns into a vapor. Quick and uniform vaporization is critical for excellent air/fuel mixture preparation and rapid, efficient combustion, especially under the transient conditions of a race.

-   Octane (C₈H₁₈): ΔH_comb ≈ –5470 kJ/mol

-   Ethanol (C₂H₅OH): ΔH_comb ≈ –1367 kJ/mol

- Secret Additives (The Fine Print): The major difference often lies in small-percentage additives (detergents, friction modifiers) that help keep the engine clean and lubricated, squeezing out every last bit of power and reliability.

How Bespoke Fuel Affects the Engine

The fuel, lubricant, and engine are developed as a single, interdependent system. The fuel's chemistry directly dictates how aggressively the engine can be tuned.

1. Detonation Resistance (The Octane Race)

- Chemistry: The specific hydrocarbon isomers and the volume of the 10% ethanol (an oxygenate) are adjusted to maximize the fuel's Research Octane Number (RON).

- Engine Effect: A higher RON means the fuel can resist premature ignition (knock) under extremely high compression and boost pressures. The fuel partner's goal is to allow the engine engineers to run a higher compression ratio and a more advanced ignition timing (firing the spark plug earlier) without the engine detonating. This is the single biggest factor in extracting more horsepower and torque.

2. Combustion Speed (Energy Release)

- Chemistry: The volatility and mix of light vs. heavy hydrocarbons in the blend determine how quickly the fuel/air mixture burns once ignited.

- Engine Effect: A fuel that burns faster and more consistently releases its energy more efficiently. This leads to higher thermal efficiency and a better torque curve, improving acceleration out of corners. Petronas and Shell often focus heavily on this aspect.

3. Thermal Management (Engine Life)

- Chemistry: The fuel and its associated lubricants play a crucial role in managing the immense heat generated in the turbo-hybrid engine. For example, some fuel components can have a slight cooling effect when they vaporize.

- Engine Effect: Better cooling and superior friction-reducing lubricants (which are developed alongside the fuel) help lower the operating temperature of critical components. This improves reliability and allows the engine to be run harder for longer periods, which is vital in a sport with strict engine component limits.

In Summary: The "Secret Sauce"

The performance difference between the fuels is not large—it's measured in fractions of a percent, maybe a few horsepower at best—but in F1, that is enough to win or lose a race.

Each fuel company optimizes the 90% hydrocarbon mix and the proprietary detergent/lubricity additives to become the perfect chemical partner for its specific engine. The resulting fuel/engine pairing is so specialized that swapping fuels between teams (e.g., Mercedes using Shell fuel) would likely result in a significant drop in performance or even cause engine failure due to improper tuning.

| Fuel Partner | Teams Supplied (Engine) | Proprietary Brand Name (F1) | Core Chemistry Focus |
| --- | --- | --- | --- |
| Shell | Ferrari, Haas (Ferrari PU) | Shell V-Power | Maximizing the thermal efficiency and combustion speed of the Ferrari V6 engine. Known for aggressive knock resistance and clean burning. |
| Petronas | Mercedes, McLaren, Williams, Aston Martin (Mercedes PU) | Petronas Primax | Deep integration with the Mercedes High-Performance Powertrains (HPP) for maximum thermal efficiency, often using advanced lubricant chemistry (Petronas Syntium) to reduce friction. |
| Mobil 1 (ExxonMobil) | Red Bull Racing, Visa Cash App RB (Honda RBPT PU) | Esso Synergy Race Fuel | Focused on high-energy density and instantaneous throttle response to match the demands of the Honda RBPT power unit under hard acceleration. |
| Castrol (BP) | Alpine (Renault PU) | Castrol EDGE | Optimization for the Renault V6 power unit, with a focus on both high power output and ensuring engine durability and heat management. |

🏎️ F1 E10 Fuel: The Composition

The term "E10" is a global standard designating a fuel blend composed of:

- 10% Ethanol (Ethyl Alcohol, C2H5OH )

- 90% Hydrocarbon Fuel (Gasoline/Petrol)

Before 2022, F1 used an E5 fuel blend (5.75% bio-components), so the mandated shift to 10% ethanol was a significant jump that required a "fundamental relook" at the entire engine design and fuel strategy.

1. The 10% Renewable Ethanol

The ethanol component is the key to the fuel's sustainability claim.

- Sourcing: The ethanol used in F1 is a second-generation biofuel. This is crucial, as it means the ethanol is not sourced from food crops like corn or sugarcane (first-generation). Instead, it must be produced sustainably from non-food biomass, such as:

  - Agricultural waste (e.g., straw, stalks)

  - Forestry waste (e.g., wood chips)8

  - Municipal waste

- Environmental Impact: Ethanol, when burned, still produces CO2. However, because the ethanol is plant-derived, the CO2 released during combustion is considered "offset" by the CO2 the plants absorbed from the atmosphere while growing. This makes the 10% portion of the fuel near net-zero carbon.

2. The 90% Hydrocarbon Base (The Performance Blend)

This 90% is what truly distinguishes F1 E10 from the E10 you pump at a local station. While the fuel must adhere to the chemical limits of commercial gasoline, it is a highly specialized blend:

- Hydrocarbon Selection: Fuel partners (like Shell, Petronas) precisely select and combine specific hydrocarbon isomers (molecules with the same chemical formula but different structures) to achieve the best performance characteristics for their engine manufacturer.

- Strict Regulation: The FIA Technical Regulations list the only acceptable compounds. Exotic, power-boosting chemicals not normally found in commercial fuel are strictly banned. This ensures the focus remains on optimization within road-relevant chemistry.

🔬 The Chemistry and Engine Effect

The inclusion of a higher percentage of ethanol forced engineers to make massive changes due to its unique chemical properties:

A. Impact on Energy and Efficiency

| Property | Ethanol vs. Gasoline | Engine Effect |
| --- | --- | --- |
| Calorific Value / Energy Density | Lower. Ethanol has less energy content per unit of mass (or volume) than pure gasoline. | Reduced Power & Efficiency Challenge. The engine has to burn more fuel (a higher Mass Flow Rate) to achieve the same energy output. Mercedes estimated the E10 change cost them a significant amount of horsepower, which they had to engineer back through combustion efficiency gains. |
| Octane Rating | Higher. Ethanol has a high Research Octane Number (RON) of around 108. | Increased Knock Resistance. The higher octane allows engine engineers to run the engine with a much higher compression ratio and more advanced ignition timing. This is the primary way the teams clawed back the lost power—by making the combustion process more aggressive and efficient without risking engine damage (knock/detonation). |

B. Impact on Combustion and Heat Management

- Oxygen Content (Oxygenate): The ethanol molecule C2H5OH contains an oxygen atom. This extra oxygen means the combustion is effectively "pre-oxygenated."

  - Effect: This helps achieve a more complete and uniform burn, which is crucial for maximizing the thermal efficiency of the V6 turbo-hybrid engine.

- Latent Heat of Vaporization: Ethanol has a very high latent heat of vaporization (it requires a lot of energy to change from a liquid to a gas).

  - Effect: As the liquid E10 fuel is injected and rapidly vaporizes inside the combustion chamber, it pulls a substantial amount of heat away from the incoming air charge. This internal cooling effect lowers the peak temperature and pressure, further increasing the engine's resistance to knock, which again allows for more aggressive engine tuning.

In short, F1's E10 fuel is a lower-energy-density fuel that is compensated for by the chemical properties of ethanol, which allows the engine to be run much more efficiently and aggressively. This makes the combined engine and fuel system a finely balanced piece of engineering.

Ending Stuff:

With the introduction of E10 fuel, Formula 1 has taken its first major step toward more sustainable energy sources. By blending bioethanol with conventional hydrocarbons, the sport maintains its performance standards while significantly lowering carbon emissions.

This development reflects a broader shift in motorsport — and chemistry — toward cleaner, more efficient, and renewable fuel technologies.

The future of fuels in Formula 1 is defined by a single, revolutionary regulation that takes effect in 2026: the mandatory use of 100% Advanced Sustainable Fuel.

This is the most ambitious fuel mandate in motorsport history and is key to F1's goal of achieving Net Zero Carbon by 2030.

🚀 The 2026 Revolution: 100% Sustainable Fuel

From 2026, the current E10 blend will be replaced entirely by a fuel that contains zero fossil carbon.

1. The Core Chemistry: e-Fuels and Biofuels

The fuel will be a mix of sophisticated hydrocarbons that are chemically identical to traditional gasoline, but derived from sustainable, non-fossil sources. This makes them "drop-in" ready for existing internal combustion engines (ICEs).

| Fuel Type | Production Source | Process |
| --- | --- | --- |
| e-Fuels (Electrofuels) | Carbon Dioxide CO2 captured directly from the atmosphere (Direct Air Capture) or industrial emissions, and Green Hydrogen (produced using renewable electricity). | The CO2 and H2 are chemically combined in a process like Fischer-Tropsch synthesis to build new hydrocarbon chains. |
| Advanced Biofuels | Non-food biomass such as agricultural waste (straw, husks), forestry residue, algae, or municipal/landfill waste. | These materials are chemically processed, fermented, or refined to create fuel-grade hydrocarbons. |

2. The Net-Zero Carbon Principle

When this fuel is burned, it still emits CO2 from the tailpipe. However, because the carbon used to create the fuel was either captured from the atmosphere (e-fuels) or was carbon that was already part of the short-term biological cycle (biofuels), there is no net addition of new CO2 to the atmosphere.

3. Impact on the 2026 Power Unit

The fuel change is synchronized with a massive overhaul of the power unit regulations:

- Equal Power Split: The engine's power will shift from a combustion-dominant design to a roughly 50% ICE / 50% Electrical split. The electrical component (MGU-K) will nearly triple its power.

- Fuel Efficiency: The new rules aim to cut the total fuel used per race by about 30% (from 100 kg to 70-75 kg). This demands the fuel be even more efficient and energy-dense.

- Performance Challenge: The primary challenge for fuel partners (like Shell and Petronas) is to create a blend that, despite the chemical constraints, still achieves the necessary high octane rating and rapid combustion speed to maintain 1000+ horsepower performance from the smaller, more electrically dependent engine.

🌎 Global Relevance: The "Drop-In" Solution

The F1 fuel is not just a niche racing technology; its ultimate goal is global impact.

The new sustainable fuel is being designed to be a "drop-in" fuel. This means it is chemically compatible with the millions of existing road cars, heavy trucks, ships, and planes that will continue to use internal combustion engines for decades to come.

By forcing rapid innovation and industrialization of e-fuels and advanced biofuels, F1 acts as a powerful development catalyst for a technology that could significantly reduce global transportation carbon emissions without requiring a complete replacement of the world's existing vehicle fleet or fuel infrastructure.